#### AMPREN3X02 Operate and monitor a continuous dry rendering process

#### Application

This unit describes the skills and knowledge required for workers to operate and monitor a continuous dry rendering process including loading raw material, operation, monitoring and shut down.

This unit applies to individuals who work under broad direction in an industrial meat rendering plant and who are responsible for operating and monitoring a continuous dry rendering process. They provide and communicate solutions to a range of predictable problems.

Work is performed in compliance with workplace procedures, according to state/territory health and safety, environmental and meat rendering regulations, legislation and standards that apply to the workplace.

##### This unit must be delivered and assessed in the context of Australian meat processing standards and regulations.

##### Licensing, legislative, regulatory requirements

No licensing, legislative or certification requirements apply to this unit at the time of publication.

#### Pre-requisite unit

Nil

#### Unit sector

Rendering (REN)

| Elements Elements describe the essential outcomes. | Performance criteria Performance criteria describe the performance needed to demonstrate achievement of the element. |
| --- | --- |
| 1. Prepare to feed cooker | 1.1 Confirm product specifications and set cooker parameters 1.2 Identify hazards, assess risks and implement control according to workplace health and safety procedures 1.3 Identify food safety hazards, assess risks and implement controls according to Hazard Analysis and Critical Control Point principles 1.4 Select and fit personal protective equipment and contamination control clothing 1.5 Check raw materials, including labels, temperature, condensate, particle size and colour, against product and workplace requirements 1.6 Undertake preoperational checks of machinery and equipment according to workplace procedures and manufacturer requirements 1.7 Clean and sanitise tools and equipment according to contamination control 1.8 Load raw materials into the cooker according to product, safety and contamination control requirements |
| 2. Start and operate continuous rendering process | 2.1 Start the cooker according to workplace operating procedures 2.2 Monitor raw material flow into the cooker maintain required feed and operating levels 2.3 Monitor and adjust cooker temperature according to product specifications 2.4 Monitor and adjust cooker outflow according to workplace requirements 2.5 Discharge rendered product from the cooker according to workplace, safety and contamination control requirements 2.6 Separate solid material from tallow according to workplace procedures 2.7 Operate meat meal size reduction equipment according to manufacturer instructions and workplace procedures 2.8 Take and test product samples to confirm rendered material meets specifications |
| 3. Monitor continuous rendering process | 3.1 Monitor and adjust variable-speed drive screw settings according to process requirements. 3.2 Monitor temperature increase to ensure required operating temperature is achieved progressively according to product and equipment requirements. 3.3 Monitor cooker shaft amps and respond to readings outside operating parameters according to workplace procedures. 3.4 Monitor sight glass levels and maintain required operating levels. 3.5 Monitor raw material feed for surges and adjust the process according to operating requirements. 3.6 Identify process or product issues and take corrective action within level of authority, or report according to workplace procedures. |
| 4. Monitor environmental controls | 4.1 Monitor vapours to ensure they are condensed according to workplace and regulatory requirements 4.2 Monitor non-condensed emissions to scrubber, bio-filter and/or after burner according to workplace and regulatory requirements 4.3 Monitor odour and other applicable environmental emissions, including wastewater, heat recovery or noise, according to workplace and environmental regulatory requirements 4.4 Monitor waste water discharge according to workplace and environmental regulatory requirements |
| 5. Shut down continuous rendering process | 5.1 Identify the appropriate shutdown procedure 5.2 Shut the process down safely according to operating procedures 5.3 Clean and sanitise rendering equipment according to workplace hygiene and contamination control requirements 5.4 Identify and report faults and maintenance requirements according to workplace procedures 5.5 Complete production and workplace records in the required format |

#### Foundation skills

| Learning | Reading | Writing | Oral communication | Numeracy |
| --- | --- | --- | --- | --- |
| 3 | 3 | 3 | 3 | 3 |

### Assessment Requirements

#### Performance evidence

An individual demonstrating competency must satisfy all the elements and performance criteria in this unit.

There must be evidence that the individual has operated and monitored a continuous dry rendering process on two separate occasions, including:

- identifying and controlling at least three different food safety hazards across the two occasions

- identifying and controlling at least three different workplace safety hazards across the two occasions

- monitoring odour and at least one of the following environmental factors:

  - wastewater

  - heat recovery

  - noise.

#### Knowledge evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

- basic operating principles of a continuous dry rendering process

- key features of HACCP principles related to continuous dry rendering processes

- workplace health and safety requirements of continuous dry rendering process

- features, benefits, limitations, and correct fit of PPE for continuous dry rendering processes

- key features of quality assurance aspects of rendered product, including:

  - colour and density of rendered products

  - end-point temperature

  - correct level of moisture content

- quality requirements of finished products including:

  - tallow moisture, insolubles and Free Fatty Acids (FFA)

  - meat meal moisture, fat content, protein, microbiological and physical contamination

  - implications on product quality and productivity of incorrect load in cooker

- effects of incorrect start up, including:

  - over shot end point temperature

  - foaming

- AS5008 Hygienic Rendering of Animal Products

- Australian Renderers Association's Code of Practice for Hygienic Rendering of Animal Products

- state and territory EPA requirements for control of non-condensable gases

- implications on product quality and productivity of incorrect load in cooker

- production problems and out-of-specification product and take corrective action

- features and functions of continuous dry rendering machinery and equipment, including:

  - manufacturer and workplace instructions for safe operation

  - control panels and ancillary controls

  - operating capacities, efficiencies and applications

  - location and purpose of guards, rails and sensors

  - feedback instrumentation

  - services required, including compressed air, electricity and water

- workplace cleaning and maintenance requirements for continuous dry rendering machinery and equipment.

#### Assessment conditions

Assessment of the skills in this unit of competency must occur in a meat rendering plant or an environment that accurately represents workplace conditions including industry or plant timeframes for production.

Assessment must ensure access to:

- raw materials required for the continuous dry rendering processes as required by performance evidence

- PPE applicable to job requirements

- tools and equipment applicable to job requirements

- organisational policies and procedures relevant to continuous dry rendering processes, operation of machinery and equipment, worker safety, contamination control, reporting and recording

- work orders or specifications used in the workplace

- a workplace supervisor.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

Mandatory workplace requirements:

Assessment of performance evidence may be in a workplace setting or an environment that accurately represents a real workplace

#### Unit mapping information

| Code and title current version | Code and title previous version | Equivalence status | Comments |
| --- | --- | --- | --- |
| AMPREN3X02 Operate and monitor a continuous dry rendering process | AMPREN302 Operate and monitor a continuous dry rendering process | Equivalent | Updated to meet 2025 Training Package Organising Framework standards. Performance and knowledge evidence requirements consistent with previous version |

### Overview information

#### Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This unit of competency was first released in AMP Australian Meat Processing Training Package Release 10.0. |