#### AMPREN3X03 Operate and monitor a low temperature wet rendering process

#### Application

##### This unit describes the skills and knowledge required for workers to operate and monitor a low temperature wet rendering process, including loading raw material, operation, monitoring and shut down.

##### This unit applies to individuals who work in an industrial meat rendering plant under broad direction and who are responsible for operating and monitoring a low temperature wet rendering process. It includes safety compliance using Hazard Analysis and Critical Control Point (HACCP) techniques, process and product quality requirements, solving problems that arise and handover of the process to others.

##### Work is performed in compliance with workplace procedures, according to state/territory health and safety, environmental and meat rendering regulations, legislation and standards that apply to the workplace.

##### This unit must be delivered and assessed in the context of Australian meat processing standards and regulations.Licensing, legislative, regulatory requirements

##### Licensing, legislative, regulatory requirements

No licensing, legislative or certification requirements apply to this unit at the time of publication.

#### Pre-requisite unit

Nil

#### Unit sector

Rendering (REN)

| Elements Elements describe the essential outcomes. | Performance criteria Performance criteria describe the performance needed to demonstrate achievement of the element. |
| --- | --- |
| 1. Prepare for low temperature wet rendering | 1.1 Confirm product specifications and set rendering equipment parameters according to workplace requirements 1.2 Identify hazards, assess risks and implement control according to workplace health and safety procedures 1.3 Identify food safety hazards, assess risks and controls according to HACCP principles 1.4 Select and correctly fit personal protective equipment and contamination control clothing 1.5 Undertake preoperational checks of machinery and equipment according to workplace procedures and manufacturer requirement 1.6 Check raw material, including cut size and colour, against product and workplace requirements |
| 2. Operate low temperature wet rendering equipment | 2.1 Operate low temperature wet rendering machinery and equipment according to workplace procedures and manufacturer requirements 2.2 Monitor and adjust heating and endpoint temperatures according to product specifications 2.3 Separate solid particles from water and tallow according to workplace requirements 2.4 Grind coarse meal to required specifications according to workplace procedures. 2.5 Dry wet solids and confirm required moisture content according to product specifications 2.6 Clean and drain equipment at the end of the production run according to workplace procedures |
| 3. Monitor low temperature wet rendering process | 3.1 Monitor the flow of material throughout the rendering process according to operating requirements 3.2 Monitor machinery and equipment to maintain optimal operating performance and adjust within level of authority 3.3 Monitor material and equipment for the presence of metal according to workplace requirements 3.4 Identify production problems and out-of-specification product and take corrective action within level of authority, or report according to workplace procedures 3.5 Check equipment for faults and maintenance requirements and report according to workplace procedures |
| 4. Monitor environmental outputs | 4.1 Monitor vapours and non-condensable gases according to workplace and regulatory requirements 4.2 Monitor emissions to scrubber, bio-filter and/or after burner according to workplace and regulatory requirements 4.3 Monitor odour and other applicable environmental outputs, including wastewater, heat recovery or noise, according to workplace and environmental regulatory requirements 4.4 Monitor waste water discharge according to workplace and environmental procedures 4.5 Identify environmental non-compliance and take action according to workplace procedures and level of authority |
| 5. Handover production run | 5.1 Verbally brief the incoming team member on production run activities and process status 5.2 Advise the team member of production, product, equipment and environmental issues requiring attention 5.3 Hand over notes, records and observations from the production run according to workplace procedures |

#### Foundation skills

| Learning | Reading | Writing | Oral communication | Numeracy |
| --- | --- | --- | --- | --- |
| 3 | 3 | 3 | 3 | 3 |

### Assessment Requirements

#### Performance evidence

An individual demonstrating competency must satisfy all the elements and performance criteria in this unit.

There must be evidence that the individual has operated and monitored a low temperature wet rendering process on two separate occasions, including:

- identifying and controlling at least three different food safety hazards across the two occasions

- identifying and controlling at least three different workplace safety hazards across the two occasions

- monitoring odour and at least one of the following environmental outputs:

  - wastewater

  - heat recovery

  - noise.

#### Knowledge evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

- principles of a low temperature wet rendering process

- workplace health and safety requirements of low temperature wet rendering process

- purpose, features and correct use of PPE required for low temperature wet rendering process

- purpose, features and correct use of clothing and equipment required for contamination control in low temperature wet rendering process

- hazard identification, risk assessment and control risk using HACCP techniques for low temperature wet rendering process

- operation and monitoring process including:

  - unusual noises in equipment which may indicate malfunction

  - equipment maintenance requirements

- key features of quality assurance aspects of rendered product, including:

  - colour and density of rendered products

  - end-point temperature

  - correct level of moisture content

- quality requirements of finished products including:

  - tallow moisture, insolubles and Free Fatty Acids (FFA)

  - meat meal moisture, fat content, protein, microbiological and physical contamination

- relevant regulatory requirements including:

- AS5008 Hygienic Rendering of Animal Products

- Australian Renderers Association's Code of Practice for Hygienic Rendering of Animal Products

- EPA requirements

- implications of noncompliance of operation and monitoring of low temperature wet rendering including:

  - incorrect temperature setting

  - discharge pump and speed of feed to the tank out of balance

  - weir liquid levels incorrectly set

  - solids from press are too wet

- features and functions of low temperature wet rendering machinery and equipment, including:

  - manufacturer and workplace instructions for safe operation

  - control panels and ancillary controls

  - operating capacities, efficiencies and applications

  - location and purpose of guards, rails and sensors

  - feedback instrumentation

  - services required, including compressed air, electricity and water

- workplace cleaning and maintenance requirements for low temperature wet rendering machinery and equipment.

#### Assessment conditions

Assessment of the skills in this unit of competency must occur in a meat rendering plant or an environment that accurately represents workplace conditions including industry or plant timeframes for production.

Assessment must ensure access to:

- raw materials required for low temperature wet rendering processes as required by performance evidence

- PPE applicable to job requirements

- tools and equipment applicable to job requirements

- organisational policies and procedures relevant to low temperature wet rendering, HACCP and food safety, workplace health and safety, contamination control, environmental monitoring, production monitoring, corrective action and handover communication

- work orders or specifications used in the workplace

- a workplace supervisor.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

Mandatory workplace requirements:

Assessment of performance evidence may be in a workplace setting or an environment that accurately represents a real workplace

#### Unit mapping information

| Code and title current version | Code and title previous version | Equivalence status | Comments |
| --- | --- | --- | --- |
| AMPREN3X03 Operate and monitor a low temperature wet rendering process | AMPREN303 Operate and monitor a low temperature wet rendering process | Equivalent | Updated to meet 2025 Training Package Organising Framework standards. Performance and knowledge evidence requirements consistent with previous version |

### Overview information

#### Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This unit of competency was first released in AMP Australian Meat Processing Training Package Release 10.0. |