#### AMPREN3X04 Operate a tallow refining process

#### Application

This unit describes the skills and knowledge required for workers to operate a tallow refining process, including preparation, rendering, separation, extraction, and shut down. It includes safety compliance using Hazard Analysis and Critical Control Point (HACCP) techniques, process and product quality requirements, and solving problems that arise with the process.

This unit applies to individuals who work under broad direction and take responsibility for their own work including limited responsibility for the work of others. They provide and communicate solutions to a range of predictable problems.

Work is performed in compliance with workplace procedures, according to state/territory health and safety, environmental and meat rendering regulations, legislation and standards that apply to the workplace.

This unit must be delivered and assessed in the context of Australian meat processing standards and regulations.

##### Licensing, legislative, regulatory requirements

No licensing, legislative or certification requirements apply to this unit at the time of publication.

#### Pre-Requisites

#### Nil

#### Unit sector

Rendering (REN)

| Elements Elements describe the essential outcomes. | Performance criteria Performance criteria describe the performance needed to demonstrate achievement of the element. |
| --- | --- |
| 1. Prepare to refine tallow | 1.1 Confirm finished tallow product specifications 1.2 Identify hazards, assess and control risks according to workplace health and safety procedures 1.3 Identify food safety hazards, assess and control risks according to HACCP principles 1.4 Select and fit personal protective equipment and contamination control equipment 1.5 Undertake preoperational checks of machinery and equipment according to workplace procedures and/or manufacturer requirements 1.6 Clean and sanitise tools and equipment according to contamination control requirements |
| 2. Extract tallow from rendered product | 2.1 Set up, start up and operate presses according to product requirements and workplace procedures 2.2 Monitor feed rate of rendered product and pressing operation to maximise safe production 2.3 Monitor product in press to ensure finished quality specifications are met and adjust equipment 2.4 Monitor and record environmental emissions according to workplace procedures |
| 3 Test tallow and unload refining equipment | 3.1 Take samples and test to ensure refined tallow product meets specifications 3.2 Identify causes of product faults and rectify according to workplace requirements 3.3 Contain spillages according to workplace and environmental requirements 3.4 Reuse under refined tallow product or discard faulty products according to workplace and environmental requirements 3.5 Transfer refined tallow product according to workplace procedures, quality and safety requirements |
| 4 Shut down tallow refining process | 4.1 Clean tallow refining equipment and report maintenance requirements 4.2 Shut the refining process down safely according to operating procedures 4.3 Identify and report maintenance requirements according to workplace procedures 4.4 Maintain workplace records in appropriate format |

#### Foundation skills

| Learning | Reading | Writing | Oral communication | Numeracy |
| --- | --- | --- | --- | --- |
| 2 | 2 | 2 | 2 | 2 |

### Assessment Requirements

#### Performance evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence the individual has set up, operated, monitored and shut down two separate industrial tallow refining processes to produce tallow from different rendered meat raw materials.

One process must be for refined liquid tallow production and one process must be for refined solid tallow production.

For each process, there must be evidence that the individual has:

- operated tallow separation and filtration equipment, including at least two of the following:

  - press or decanter

  - screening

  - settling tanks

- monitored temperature and made adjustments according to work instructions and product specifications

- monitored environmental emissions according to workplace and Environmental Protection Agency (EPA) requirements, including odour control and one of the following:

  - waste water

  - oil and grease

#### Knowledge evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

- basic principles of tallow refinement including liquid tallow and solid tallow

- key features of Hazard Analysis and Critical Control Point (HACCP) principles related to tallow refinement

- workplace procedures for safe movement of workers and visitors in and around plant and tallow refining machinery and equipment

- features, benefits, limitations, and correct fit of PPE for refining tallow

- quality requirements of rendered materials that are used to produce refined tallow, including:

  - temperature

  - particle size

- product requirements, including:

  - free fatty acid levels

  - colour

- Australian Standards for Hygienic Rendering of Animal Products

- Australian Renderers Association's Code of Practice for Hygienic Rendering of Animal Products, including cleaning and sanitation of tallow refining equipment

- codes and standards that apply including EPA requirements for monitoring environmental emissions, including odour control, waste water, heat recovery and noise as applicable

- features and functions of tallow processing and refining machinery and equipment, including:

  - manufacturer and workplace instructions for safe operation

  - control panels and ancillary controls

  - operating capacities, efficiencies and applications

  - location and purpose of guards, rails and sensors

  - feedback instrumentation

  - services required, including compressed air, electricity and water

  - shut down

- workplace recording and reporting requirements for tallow refinement.

#### Assessment conditions

##### Assessment of the skills in this unit of competency must occur in a meat rendering plant or an environment that accurately represents workplace conditions including industry or plant timeframes for production.

Assessment must ensure access to:

industrial tallow refining machinery and equipment required for the tasks specified in the performance evidence.

raw materials specified in the performance evidence

PPE and contamination control equipment clothing required for the tasks specified in the performance evidence

workplace procedures for refining tallow and operation of machinery and equipment stipulated in the performance evidence

workplace procedures for worker safety and contamination control

reporting and recording formats

##### Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

##### Mandatory workplace requirements:

Assessment of performance evidence may be in a workplace setting or an environment that accurately represents a real workplace.

#### Unit mapping information

| Code and title current version | Code and title previous version | Equivalence status | Comments |
| --- | --- | --- | --- |
| AMPREN3X04 Operate a tallow refining process | AMPREN304 Operate a tallow refining process | Equivalent | Updated to meet 2025 Training Package Organising Framework standards. Performance and knowledge evidence requirements consistent with previous version |

### Overview information

#### Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This unit of competency was first released in AMP Australian Meat Processing Training Package Release 10.0 |